- DATA BUOY CO-OPERATION PANEL

- The Low-cost Barometer Drifter

By the Technical Co-ordinator of the Data Buoy Co-operation Panel

(November 1993)

1) Introduction

Since its third session, the DBCP has been increasingly involved in efforts

to persuade meteorologists, and oceanographers to collaborate on combined

meteorological and oceanographic drifting buoys.

Developments have been conducted in two directions of collaboration: (i) to

install thermistor strings onto non-Lagrangian regular meteorological buoys

(developments conducted by meteorologists at M t o-France), and (ii) to equip

standard SVP Lagrangian drifters with barometer ports (developments conducted

by oceanographers at the Global Drifter Center). Both developments are

successful. The two devices are complementary since we now have buoys capable

of measuring wind and sub-surface temperature profiles on one hand, and buoys

capable of measuring sea surface currents on the other hand. Both devices

measure Atmospheric  Pressure and Sea Surface Temperature. However the cost

of the meteorological buoys equipped with thermistor strings remains

relatively high. This article only deals with the Lagrangian drifter equipped

with a barometer port (refer to the DBCP annual report for 1992 for more

details regarding the other device).

The Global Drifter Center (GDC, at Scripps Institution of Oceanography, La

Jolla) of the WOCE and TOGA Global Surface Velocity Program (SVP) was

responsible for the development of a low cost Lagrangian Drifter equipped

with a barometer port. The DBCP collaborated actively with the GDC in the

field test of some 25 prototypes "barometer" drifters. Meteorological Agencies

of Australia, Canada, France, and the United Kingdom, as well as the GDC

purchased and deployed 25 units, including a  total of 16 units deployed at

sea during the period August 1992 to February 1993 (figures 6 and 7).

The NOAA National Ocean Service in collaboration with the Scripps Institution

of Oceanography are now confident enough in the new design of the low cost

barometer drifter to purchase and plan for deployment in 1994 of 86 units in

the Southern Hemisphere as part of the WOCE and TOGA programs. Commercial

production will start in late 1993 with a purchase cost of US$ 4600 per unit.

At its ninth session in Athens, 19-22 October 1993, the Data Buoy Cooperation

Panel recognized that the design was successful. It stressed that this new

situation opens the door for direct cooperation between meteorologists and

oceanographers particularly because the design meets both communities

requirements, and because it is much less expensive than previous designs of

buoys measuring atmospheric pressure. For example implementation of common

oceanographic and meteorological buoy programs would be very cost effective

for both communities and would therefore make it possible for the same cost

to dramatically increase the data return. Such rationalized and standardized

programs could then very well be integrated in the Global Ocean Observing

System (GOOS) and the Global Climate Observing System (GCOS).

The DBCP therefore urged its participants, and all others involved in drifting

buoy programmes to look as much as possible for opportunities of co-operation

at the national level between oceanographers and meteorologists. It is to

assist them in this task that the DBCP decided to include this article in the

present DBCP annual report for 1993.

2) Design of the low cost "barometer" drifter:

2.1) Background:

The low cost barometer drifter is basically a standard SVP drifter to which

an air pressure port has been added (figure 2, figure 3). The standard SVP

drifter (figure 1) is now a proven and reliable design and it has been deployed

at sea in large quantities for oceanographic research programs as part of the

World Ocean Circulation Experiment (WOCE) and the Tropical Ocean and Global

Atmosphere programme (TOGA). For the period 1 July 1991 to 31 January 1993,

the WOCE Surface Drifter Data Assembly Centre has processed data from 1315

drifters (WOCE Report No. 104/93) deployed in the Pacific and North Atlantic

oceans. It is capable of accurately measuring sea surface currents (+/- 1 cM/S)

in 10 M/S winds and sea surface temperature (+/- 0.1 C). Nominal life time is

18 month. It has been shown that half life time of standard SVP drifters is

in the order of 440 days (Figure 5).

2.2) Surface current measurement

For measuring Surface Velocity, standard SVP buoys have been designed to be

good Lagrangian drifters (buoys which follow the water motion well) and very

specific requirements of drogue and surface float design have been developed

(large holey sock drogue, spherical floats and thin wire tethers...). Laboratory

and at sea tests have been conducted to guarantee the reliability of SVP

drifter measurements.

The slip (i.e. the motion of the centre of the drogue relative to the moving

water parcel) has been minimized. Many phenomena can induce slip; the main

ones are wind stress, surface gravity wave effects and vertical shear of

currents. Therefore tests have been conducted on various shapes of floats and

drogues (NOAA data report 1990). These tests show that the most efficient

shapes are small, spherically-symetric surface and subsurface floats,

thin-wire tethers and a large semi-rigid drogue. The drogues which have a

high drag coefficient and stable water following characteristics are the

TRISTAR (Niiler, et al., 1987) and the Holey Sock (Nath, et al., 1979). The

drag area ratio is the drag coefficient of the drogue times the frontal area

divided by the sum of the products of the drag coefficient and the largest

projected frontal areas of floats and tethers. A drag area ratio for the

drifter greater than 40 will give the instrument the capability to make

current measurements accurate to within 2 cM/S. Using a correction formula,

a wind correction will then improve this accuracy to 1 cM/S if the wind is

known within 4 M/S (Figure 10). In extra tropical areas, if an optimized

network of low-cost barometer drifters is maintained, the air pressure field

and consequently the wind field will be known to a better accuracy.

The same general design as for the standard SVP Lagrangian drifter has been

chosen for the low-cost barometer drifter.

2.3) Drogue detector (Submersion switch)

A drogue detector is necessary for ascertaining if the drogue is still

attached. A Drifter without a drogue, is of little value for surface

velocity measurements. Since the surface float goes under the water more

often when the drogue is attached, one principle is to install a submersion

detector (switch) on the surface float and to analyze the time series in

order to deduce if the drogue is still attached.

2.4) Sea Surface Temperature measurement

The low cost "barometer" drifter is also equipped with a Sea Surface

Temperature sensor that is designed to make measurements accurate to 0.1

Celsius. Once again, experience gained with the standard SVP drifter has been

used. To obtain this accuracy, tests show that one must install the

temperature sensor outside the hull of the drifter float. Also, calibrations

of a number of thermistors while connected to the electronics circuitry in a

test tank in various range of temperatures must be done. Only these kind of

tests and calibrations can provide accurate coefficients to be used to

convert raw data (resistance) into physical values (Celsius) within +/- 0.1

Celsius. The life time of the sensor will exceed that of the transmitter.

2.5) Atmospheric Pressure Measurement

The air pressure port has been designed to withstand frequent immersion with

no loss of accuracy.  The port is  elevated to some height above the float

itself to avoid Venturi effects caused by air flow over the curved float

surface. The total surface of the mast is lower than 10% of the total frontal

area so that wind stress does not induce a substantial slip effect compared

to the one induced through the hull itself. The design is based on a port

used on moored buoys by the United Kingdom Meteorological Office, which has

had extensive field tests in the wind tunnel. Internal baffling is provided

against submergence surges and sufficient back up volume of air assures that

water does not enter the barometer duct.

The barometer port design as shown in figure 4, is based on the following

rationale (WOCE/TOGA Lagrangian Drifter with Barometer Port, May 1991):

(i) Field observations indicate that the surface float of the SVP

Lagrangian drifter is pulled under the water to a depth of 1-2 m

at the crests of wind waves, therefore an overpressure of 200 hPa

can be expected on the barometer. Data from the submergence switch

on drifters in WOCE Heavy Weather Drifter Test (Sybrandy and Niiler,

1991) indicate that they spend about 20-30% of the time under the

water in winds in excess of 15 m/s. Upon resurfacing, the port has to

clear from sea-water quickly and completely. Flaps and valves to close

a port will fail or become encrusted. An inverted port, with sufficient

backup volume of air which can be compressed upon submergence so the

water is kept out of the barometer air duct was incorporated in the

design.

(ii) A long air pressure duct to the barometer can collect condensation in

the extreme changes of moisture and temperature which occur in

synoptic weather systems. This problem was solved by placing the

barometer very close to and above the air intake. Specially configured

barometers were made for this application for GDC by several

manufacturers.

(iii) In a wind stream, the surface float produces a lowering of air

pressure due to the Bernouilli effect. In 10 m/s wind, this effect

produces less than 0.1 hPa pressure lowering at a distance of one

radius of a sphere. The barometer port air intake is placed on a mast

24 cm above the top of the sphere. A second Bernouilli effect is

produced by the airflow around the mast. This problem has been studied

extensively, and a tabular wind shield, with air intake holes inside

an inserted, second sleeve is adopted (Osmund and Painting, 1984).

(iv) The sampling and averaging scheme for the air pressure has to be

sensitive to when the port is under the water. Tests have run at sea

under 15 m/s wind conditions off San Diego, Ca. (WOCE/TOGA Lagrangian

Drifter with barometer port, May 91, Sybrandy and Niiler) where

pressure was sampled at 2Hz inside the surface float. A laboratory

standard barometer of identical construction was used to obtain data

at identical rates about 3 meters above sea level in a semi-enclosed

laboratory on a ship. No significant wind effect, or delay times, were

observed on the barometer port response on the surface float in the

water.

The sensor itself is an AIR SB-1A model. It is a ceramic diaphragm capacitance

sensor equipped with a built-in temperature compensating circuit. AIR sensors

have been carefully tested for WOCE and finally proved reliable (Payne et al,

IMET). Accuracy is +/- 1 hPa with a stability of +/- 1 hPa over a one year

period. Sensor output is digital in tenth of hPa.

In the latest scheme (proposed at the joint DBCP-SVP workshop 4-6 May 1993),

data are sampled at 1 Hz, and averaged over a 80 seconds period. A dedicated

despiking algorithm was designed to remove from the average these air

pressure measurements made while the barometer port is submerged:

"The algorithm will first average the lowest 20 of 80 measurements; it will

then throw away all measurements within the entire 80 measurement set with

values greater than 1 hPa over that average, and transmit the median point

of the remaining values."

The latest average of every hour is stored on-board. The last 24 hourly

measurements are memorized on-board and transmitted through Argos using

multiplexing techniques. It is expected that the full serie of 24 hourly

measurements will be recovered every day. Hence the latest available air

pressure and tendency measurements (real time) as well as the synoptic air

pressure measurements will be distributed on GTS (deferred-time).

3) Field tests:

The Data Buoy Co-operation Panel participated actively in the testing of a

total of 25 prototype Barometer drifters (MOD-1):

*  The Atmospheric Environment Service purchased 3 units and deployed them

in December 1992 in the North East Pacific Ocean.

*  The Australian Bureau of Meteorology purchased 3 units and deployed 2

in February 1993 in the Tasman Sea.

*  The Global Drifter Center purchased 11 units and deployed 3 units in

August 1992 and 4 units in January 1993 in the California Currents

system.

*  Meteo-France purchased 3 units and deployed them in in August 1992 in

the Golfe de Gascogne.

*  The United Kingdom Meteorological Office purchased 5 drifters and

deployed 4 in the North Atlantic Ocean.

A joint DBCP-SVP workshop was held 4-6 May 1993 in San Diego in order to

evaluate the quality of the prototypes and to propose design changes (SIO Ref

Series 93/28, WOCE report 108/93). At the time of the meeting, 16 prototypes

had been deployed at sea. In general, despite limited success with some of

the buoys, the test participants were pleased with the performance of the

SVP drifter fitted with barometer. In particular it was demonstrated that

the quality of pressure data in general was as good as for regular FGGE type

meteorological buoys (see figures 8 and 9).

The meeting agreed that the main problems detected with the first 16

prototypes deployed at sea were: (i) Through hull connector failure,

(ii) Upper hemisphere failure (lack of fiberglass), and (iii) Despiking

algorithm problem. These problems are believed to have caused premature

death for 6 out of the 16 prototypes. In order to hopefully show the eventual

reliability of the system, the meeting proposed some design modification (for

MOD-2):

Hardware modification: to replace the high power Lithium batteries with

Alkaline batteries, to increase the hull diameter, to reinforce the new hull,

to improve the hemisphere sealing, to increase the diameter of the port attach,

and to improve the strength of the SST probe;

Software modifications: to change the Argos message format, and to improve

the "despiking algorithm.

Another field test was proposed in order to validate the decided design

changes.  New prototypes (MOD-2) have already been shipped to the

participants and some have been deployed:

*  The Atmospheric Environment Service of Canada deployed 2 units in

September 1993 in the North-West Pacific Ocean;

*  The Australian Bureau of Meteorology will deploy 2 units at the end

of November 1993 in the Tasman Sea;

*  The Global Drifter Center deployed 5 units in September 1993 in the

California currents System;

*  Meteo-France deployed 1 unit in September 1993 in the Golfe de Gascogne;

*  The UKMO will deploy 2 units at the end of November 1993 in the North

Atlantic Ocean.

4) Production and commercialization:

Since the NOAA National Ocean Service in collaboration with the Scripps

Institution of Oceanography purchased 86 low-cost barometer drifters to be

deployed in 1994 in the Southern Ocean as part of the TOGA and WOCE programmes,

the commercialization process already began. Orders were placed with two

manufacturers at a rate of $4600 per unit:

Technocean Associates

4422 SE 9th Ave

Cape Coral, FL 33904

USA

Tel: (+1) (813) 945 7019

Fax: (+1) (813) 574 5613

Clearwater Instrumentation

49 Walnut Park, Building No 2

Wellesley Hills, MA 02181

USA

Tel: (+1) (617) 239 3305

Fax: (+1) (617) 239 3314

Early 1995, the Global Drifter Center will publish a construction manual for

the low-cost Barometer drifter for distribution to the oceanographic and

meteorological communities. Copies of the manual may be obtained from

Pr. Pearn P. Niiler, Global Drifter Center at Scripps Institution of

Oceanography, La Jolla, California 92093, USA (Fax: (+1) 619 534 7931).

5)  Opportunities for Cooperation between Oceanographers and Meteorologists:

For the reasons detailed below, the low-cost Barometer drifter is a device

which can be shared by both oceanographic/research and meteorological/

operational communities:

-1- For oceanographers (research):

* Good water following characteristics:

- High drag area ratio (>40)

- Low wind stress (small spherical hull etc...)

* Long life time (half life = 440 days)

* Capability of measuring Sea Surface Temperature

* Capability of measuring Air Pressure (a good Air Pressure field leads to

a good wind field which in turn can be used to substantially deduce wind

stress from computed surface velocities).

* Series of hourly data are available. Normally the system should be able

to recover all the 24 hourly data collected by the platform every day

(redundancy).

-2- For meteorologists (operational):

* Stays in a given area longer (the drifter is drogued at 15 meters)

* Long life time

* Capability of measuring Air Pressure

* Capability of measuring Sea Surface Temperature

* Data distributed on GTS in real time

* Hourly data are measured. Hence synoptic as well as the latest measured

data are available (real time). Redundant data are filtered.

The cost of the Lagrangian drifter equipped with a barometer port (less than

US$5000) is well below the cost of a regular FGGE type meteorological buoy

(in the order of US$15000) which is capable of measuring the same physical

variables (SST, Pressure) plus Air Temperature. Life time and quality of

measurements of both devices are very similar. The DBCP recognized at its

eighth session (DBCP-VIII, Paris, Oct. 92) that Air Temperature measurements

from buoys were not especially reliable. Hence FGGE type meteorological buoys

remain competitive only when capable of measuring addition physical variables

such as wind and sub-surface temperature profiles (cost > US$20000 in that

case). In addition, it has been shown that wind data from drifting buoys can

be better than these from ships (DBCP annual report for 1992).

Hence to be cost-effective and to satisfy a wide community of users, a drifting

buoy program should ideally only include a mixture of standard SVP drifters

(surface currents, SST), low cost barometer drifters (surface currents, SST,

Air Pressure), wind buoys (SST, Wind, Air Pressure) and wind buoys equipped

with thermistor strings (SST, Wind, Air Pressure, Sub-surface temperature

profile): Wind buoys and standard SVP drifters could for example be deployed

inside the tropics where the atmospheric gradient is weak (except in tropical

cyclonic systems) and geostrophic relationship is not valid near the equator.

Barometer drifters could be deployed in large quantities outside the tropics.

Thermistor strings wind buoys would be deployed wherever an express need for

sub-surface temperature profiles (0 to 150 meters depth) is required.

Conclusion: As demonstrated on the example presented in Annex A, based on 1993

commitments, it is clear that both oceanographic and meteorological communities

would benefit from a large scale cooperation based on new co-operative drifting

buoy programmes provided that these programmes principally rely upon the

low-cost barometer drifter. If desired, such programmes could be placed

under the GOOS and GCOS umbrellas. It is basically at the national level

that such steps should be taken although the DBCP can assist meteorological

and oceanographic agencies to collaborate at the international level.

Annex A: Example of cooperation meteorologists/oceanographers

In 1993 oceanographers maintained (basically under the WOCE programme) a

network of approximately 550 regular Lagrangian drifters (SST and surface

currents only) in various ocean areas. In order to save Argos costs,

approximately 380 of these drifters transmitted only 1/3 of the time

through the system. Hence a total of about 297 PTT.Years were purchased

from Service Argos:

Hardware cost                550 * $4000   $ 2 200 000

Argos costs                  297 * $4000   $ 1 188 000

Approximate Total cost for oceanographers  $ 3 388 000

In 1993 meteorologists maintained approximately 180 meteorological buoys

in various ocean areas. All buoys reported full on via Argos:

Hardware cost                180 * $15000  $ 2 700 000

Argos costs                  180 * $4000   $ 720 000

Approximate Total cost for meteorologists  $ 3 420 000

Of course, since the buoys did not meet both communities requirements,

oceanographers could only use the data from oceanographic buoys (550 buoys)

and meteorologists could obtain GTS pressure data only from meteorological

buoys (180 PTT.Year of Pressure data).

For our example, based on the same level of commitments than for 1993, below

is detailed the approximate data return that each community could have expected

if both communities had collaborated in 1993 on common programmes.

In their different drifting buoy programmes, both communities made commitments

for approximately $ 6 808 000. Suppose that for the same cost they had planned

to deploy a mixture of 5% meteorological wind buoys and 95% of low-cost

barometer drifters as part of common drifting buoy programmes.

For a total of N buoys, hardware cost would have been:

0.05*N*$20000 + 0.95*N*$5000.

If the buoys were reporting full on through Argos, Argos cost would have been:

N*$4000.

The equation is therefore $ 6808000 = N * $ 9750

Hence for the same cost they would have deployed a total of 698 buoys,

i.e. 663 low-cost barometer drifters and 35 wind buoys.

Benefits for oceanographers would have therefore been:

- 113 additional buoys capable of measuring sea surface currents

- All the 663 buoys would report full on through Argos (instead of

1/3 for 380 of these)

- All the buoys would be equipped with a barometer port therefore

improving the accuracy of the wind field analysis outside the tropics

therefore improving the accuracy of the computed surface velocities.

Benefits for meteorologists would have been:

-  518 additional PTT.Year of Air Pressure data.

-  35 wind buoys (versus less than 10 in 1993)

-  All the operating buoys would be reporting on GTS (since this is a

co-operative programme)

Annex B: Figures

Figure 1: The standard SVP drifter and surface float Close-up

Figure 2: The low-cost Barometer Drifter Surface Float (MOD-1 prototype, the

new MOD-2 is a 16" reinforced hull with an increased diameter for

the port attach and with Alkaline battery pack)

Figure 3: Photograph of a fully-assembled 1992 low-cost Barometer drifter

(MOD-1).

Figure 4: Schematic of the Barometer Port designed by GDC. The air path goes

through a wind shield, two baffles, Goretex shields, and into a

long tygon tube. (Courtesy of GDC).

Figure 5: Survivability of standard SVP drifters deployed in late 1992 and

in 1993 (Courtesy of GDC).

Figure 6: Trajectories for M t o-France prototypes deployed in August 1992.

Figure 7: Trajectories for GDC prototypes deployed in August 1992 in the

California Currents from August 92 to November 92

Figure 8: Monitoring Statistics for M t o-France prototypes deployed in

August 1992.

Figure 9: Trajectories and intercomparison between SVP Barometer drifter 1356

and FGGE type meteorological buoy Marisonde 5794 soon after August

1992 deployment.

Figure 10: A model for the interpretation of slip data: Downwind Model

Us=a*W/R + b*D/R, Crosswind Model Us=c*D/R. A comparison of the

modeled and observed slip of drifters down-wind (upper panel)

and crosswind (lower panel). The slip units are in cM/S. The least

square fits give a=3.8, b=9.5, c=10.2, where W is the wind speed

in M/S, D is the measured velocity difference across the drogue in

cM/S and R is the drag area ratio. This model accounts for 87% of

the variance of the observed data.

References :

WMO-IOC DBCP III session report, Oct 1987, Geneva, Switzerland.

WMO-IOC DBCP VIII session report, Oct. 1992, Paris, France.

WMO-IOC DBCP IX session report, Oct. 1993, Athens, Greece.

WMO-IOC DBCP annual report for 1992.

WCRP-26 Surface Velocity Programme (SVP). Workshop report of WOCE/SVP Planning

Committee and TOGA Pan-Pacific Surface Current Study, Miami; Florid, USA,

25-26 April 1988.

WOCE and TOGA Surface Velocity Programme Planning Committee, Report of the

fifth meeting; SVP-5, Hamilton, Bermuda 6-8 April 1992; WOCE Report No. 87/92;

September 1992 WOCE  International Project Office, Wormley.

University Of California, Scripps Institution of Oceanography, La Jolla,

California 92093. The WOCE/TOGA SVP Lagrangian Drifter Construction Manual by

Andrew L. Sybrandy and Pearn P. Niiler; February 1991; Sponsored by the

National Science Foundation SIO Reference 91/6; WOCE Report Number 63.

Osmund, A. and D.J. Painting (1984) Static pressure heads for meteorological

se. United Kingdom Meteorological Office, Tecemo/Reference III.15, WMO

Technical Conference on Instruments and Cost Effective Meteorological

Observations.

Poulain P.M, J.D. Illeman, and P.P. Niiler, Drifter Observations in the

California Current System,1985-1986, Data Report, SIO Ref. 87-27, Scripps

Institution of Oceanography, University of California, San Diego.

Poulain P.M. and P.P. Niiler, Statistical Analysis of the Surface Circulation

in the California Current System Using Satellite-Tracked Drifters, Oct. 89,

Journal of Physical Oceanography, Vol. 19, No. 10.

Niiler P.P., Russ E. Davis, and Henry J. White, 1987: Water-following

characteristics of a mixed layer drifter. Deep-Sea Research, Vol. 34, No. 11,

pp. 1867-1881.

WOCE surface flux determinations, A strategy for in situ measurements for

fluxes, Report of the working group on in situ measurements for fluxes,

Scripps Institution of Oceanography, Part one, 27 Feb.-3 Mar. 89.

Nath J. et al.,Hydrodynamic coefficients for flexible drogues, Published by

the American Society of Civil Engineers,Proceedings of the Specialty

Conference Civil Engineering in the Oceans IV vol. II, San Francisco

September 10-12 1979.

Payne, R.E., G.H. Crescenti and R.A. Weller, 1989 Improved Meteorological

Measurements from Buoys and Ships (IMET) : Preliminary Report on Barometric

Pressure Sensors WHOI Ref. No. 89-49.

NOAA Data Report ERL AOML-17, Drifting Buoy Intercomparison Test Results,

David S. Bitterman, Peter P. Niiler,  Yanick Aoustin, Amaurdic du Chaffaut,

AOML, Miami, Florida, August 1990.

Sybrandy, A. and P.P. Niiler, 1991. WOCE Heavy Weather drifter tests -

Technical Report. Scripps Institution of Oceanography.

WCRP/WOCE Summary of Resource Commitments, Satellites - Sea Level - Moored

Arrays - Floats -Drifters - WHP - XBTs - Facilities. WOCE report No. 64/91,

March 1991, WOCE International Project Office, Wormley.

WOCE/TOGA Lagrangian Drifter with Barometer Port, May 1991, Andrew L.

Sybrandy and Pearn P. Niiler, Global Drifter Center, Scripps Institution of

Oceanography, Sponsored by the National Science Foundation.

WOCE Data Management; Data Sharing Policy and Practices, Data Assembly and

Analysis Centres, Satellite Data Availability and Data Information Unit; WOCE

Report No. 104/93; April 1993, WOCE International Project Office, Wormley.

Drifting Buoy Co-operation Panel; Low-cost Meteorological Measurements from

Data Sparse Ocean Areas; WMO-IOC November 1991.

Scripps Institution of Oceanography Reference Series; SIO Series 93/28; WOCE

Report 108/93. Surface Velocity Program / Drifting Buoy Co-operation Panel

Workshop on evaluation of low cost Barometer drifters.