Evaluation of Iridium SVP-B drifters

News from Météo-France trials
and new version of the transmission format

By Pierre Blouch, Météo-France

April 4th, 2007

The first Iridium SVP-B prototype deployed last year (Id. 508040 - WMO 62508) was voluntarily recovered and sent to Metocean for expertise, refurbishment and improvement. It is now almost ready with three more other drifters.

The second prototype (Id. 509040 - WMO 62509) is still correctly transmitting reliable SST values onto the GTS, 5 months after its deployment.

The data format has been changed. It takes into account the remarks done at the end of the previous paper describing it. The new format, described on next page, should be this of operational drifters. Main changes include:

- sensors order has been reorganized to take into account some drifters could have no GPS in the future if Iridium positions are used;

- checksum has been removed. Not used since the integrity of Iridium messages is ensure by the system,

- word length for “mode” increased,

- word length for battery voltage increased,

- word length for submergence decreased,

- transmission of GPS fix times has been added to be able to correctly sent it onto the GTS (FM18 BUOY messages).

The number of attempts to send an Iridium message has not been introduced in the message format. On the SVP-B prototypes, each attempt lasts 5 minutes. In case the transmission doesn’t succeed during these 5 minutes, a new attempt is performed one hour later for the same data. Consequently, the number of attempts may be deduced from the transmission delay. A message having a 1-hour delay was unsuccessfully sent after a first unsuccessful attempt; the transmission of a message having a 2-hour delay failed twice, etc....

The GTS data processing will be modified accordingly. Météo-France will be glad to process and to report onto the GTS the data of buoys which will use it.

The operating principles – one observation every hour and one GPS position 8 times a day – are unchanged. The start of the transmission is still done at HH+11 minutes.

Two drifters should be deployed within a few weeks in the North Atlantic. Two others will be deployed in the Arctic in August. In addition, Météo-France is ordering four new SVP-B drifters to Metocean and two others to Marlin. In parallel, E-SURFMAR would agree that 5 out of the 45 drifters which will be soon ordered be “Iridium upgraded” by the DBCP fund.

Format (version 3 – used on the 1+3 new prototypes)

| # Parameter | Bits | Pos | Min | Max | Formula |
| --- | --- | --- | --- | --- | --- |
| Mode | 3 | 0 | 0 | 7 | Fixed = 3 (0011) |
| Observation time since 1st of January at 00:00 | 16 | 3 | 0 | 16383 | Hour (UTC) = n * 0.25 |
| Air pressure | 11 | 19 | 850 | 1054.7 | AP (hPa) = n*0.1 + 850 |
| SST | 9 | 30 | -5 | 35.88 | SST (°C) = n*0.08 - 5 |
| Pressure tendancy | 9 | 39 | -25.5 | 25.6 | dP (hPa) = n*0.1 - 25.5 |
| Submergence count | 6 | 48 | 0 | 100 | Subm. (%) = n * 1.6129 |
| Battery voltage | 6 | 54 | 10.75 | 17.15 | Vbat (V) = n*0.1 + 10.75 |
| Iridium transmission duration | 8 | 60 | 0 | 255 | SBDT (s) = n |
| GPS fix time since 1st of January at 00:00 | 16 | 68 | 0 | 16383 | Hour (UTC) = n * 0.25 |
| GPS Latitude | 20 | 84 | -90 | 98 | Lat (deg) = n*0.00018 - 90 |
| GPS Longitude | 21 | 104 | -180 | 197 | Lon (deg) = n*0.00018 - 180 |
| Time to first GPS fix | 7 | 125 | 0 | 254 | TTFF (s) = n * 2 |

Columns :

- Mode is used to describe the type of data (Meteo-France convention - “3” is used for drifting buoys).

- Column “Bits” : length of the word in bits for the parameter;

- Column “Pos” : Start bit in the message for the parameter;

- For some parameters, “Max” is just an approximation;

- Total length of the message : 16.5 bytes.

By convention, the 6 last digits of the Iridium transmitter Id (IMEI) are used to identify the drifter
     equipped with it.