Iridium SVP-BS transmission format

Version 1.1

By Pierre Blouch, Météo-France

February 12th, 2009

| # Parameter | Bits | Pos | Min | Max | Formula |
| --- | --- | --- | --- | --- | --- |
| Mode | 3 | 0 | 0 | 7 | Fixed = 3 (011) |
| Observation time since 1st of January at 00:00 | 16 | 3 | 0 | 16382 | Hour (UTC) = n * 0.25 |
| Air pressure | 11 | 19 | 850 | 1054.6 | AP (hPa) = n*0.1 + 850 |
| SST | 12 | 30 | -5.00 | 35.94 | SST (°C) = n*0.01 - 5 |
| Pressure tendancy | 9 | 42 | -25.5 | 25.5 | dP (hPa) = n*0.1 – 25.5 |
| Submergence count | 6 | 51 | 0 | 100 | Subm. (%) = n * 1.6129 |
| Battery voltage | 6 | 57 | 10.75 | 16.95 | Vbat (V) = n*0.1 + 10.75 |
| Iridium transmission duration | 8 | 63 | 0 | 254 | SBDT (s) = n |
| Salinity | 12 | 71 | 15.00 | 55.94 | ###### SSS (psu) = n*0.01 + 15 |
| CT Temperature | 12 | 83 | -5.00 | 35.94 | ###### CT_Temp (°C) = n*0.01 - 5 |
| GPS fix time since 1st of January at 00:00 | 16 | 95 | 0 | 16382 | Hour (UTC) = n * 0.25 |
| GPS Latitude | 20 | 111 | -90 | 98 | Lat (deg) = n*0.00018 - 90 |
| GPS Longitude | 21 | 131 | -180 | 197 | Lon (deg) = n*0.00018 - 180 |
| Time to first GPS fix | 7 | 152 | 0 | 126 | ###### TTFF (s) = n*2 |
| CT Sensor Error | 1 | 159 | 0 | 1 | ###### Err = n |

Table 1 - Dataformat

Columns :

- Mode is used to describe the type of data. It serves to determine which format must be used for GTS data. Meteo-France convention: “3” is used for buoy data; “1” for ship data.

- Column “Bits” : length of the word in bits for the parameter;

- Column “Pos” : Start bit in the message for the parameter;

- For some parameters, “Max” is just an approximation;

Other remarks :

- Values for missing sensor or computation should be “all bits fitted to 1”;

- Total length of the total version 1.1 message : 160 bits.