Iridium SVP-B transmission format

Version 4

By Pierre Blouch, Météo-France

February 12th, 2009

| # Parameter | Bits | Pos | Min | Max | Formula |
| --- | --- | --- | --- | --- | --- |
| Mode | 3 | 0 | 0 | 7 | Fixed = 3 (011) |
| Observation time since 1st of January at 00:00 | 16 | 3 | 0 | 16382 | Hour (UTC) = n * 0.25 |
| Air pressure | 11 | 19 | 850 | 1054.6 | AP (hPa) = n*0.1 + 850 |
| SST | 9 | 30 | -5 | 35.8 | SST (°C) = n*0.08 - 5 |
| Pressure tendancy | 9 | 39 | -25.5 | 25.5 | dP (hPa) = n*0.1 - 25.5 |
| Submergence count | 6 | 48 | 0 | 100 | Subm. (%) = n * 1.6129 |
| Battery voltage | 6 | 54 | 5 | 17.4 | Vbat (V) = n*0.2 + 5 |
| Iridium transmission duration | 8 | 60 | 0 | 254 | SBDT (s) = n |
| 2nd Iridium Tech. parameter | 8 | 68 | 0 | 254 | ###### See table 2 |
| GPS fix time since 1st of January at 00:00 | 16 | 76 | 0 | 16382 | Hour (UTC) = n * 0.25 |
| GPS Latitude | 20 | 92 | -90 | 98 | Lat (deg) = n*0.00018 - 90 |
| GPS Longitude | 21 | 112 | -180 | 197 | Lon (deg) = n*0.00018 - 180 |
| 1st GPS Technical parameter | 7 | 133 | 0 | 126 | ###### See table 2 |
| 2nd GPS Technical parameter | 4 | 140 | 0 | 14 | See table 2 |

Table 1 - Dataformat

Remarks :

- Mode is used to describe the type of data. It serves to determine which format must be used for GTS data. Meteo-France convention: “3” is used for buoy data; “1” for ship data.

- Column “Bits” : length of the word in bits for the parameter;

- Column “Pos” : Start bit in the message for the parameter;

- For some parameters, “Max” is just an approximation;

- Values for missing sensor or computation should be “all bits fitted to 1”;

- Total length of the total basic version 4 message : 144 bits i.e.18 bytes;

- Message length may be shortened if the last parameters are not used. For instance, a SVP-B drifter having no GPS may have 8-bytes long messages.

|  | 2nd Iridium parameter | 1st GPS parameter | 2nd GPS parameter |
| --- | --- | --- | --- |
| ##### Marlin | Iridium transm. retry | Time to first GPS fix | Nb of GPS satellites |
| Metocean | Unused | Time to first GPS fix1 | Unused |
| Pacific Gyre | Unused ? | Time to first GPS fix1 ? | 2D or 3D GPS fix |

Table 2 – Examples of manufacturer’s choices for technological parameters

TTFF(s) = n * 2